Ubuntu Innovations (Pty) Ltd

Week 2 — Task 5: IP Addressing Plan

IT Infrastructure Capstone Project

# 1. Design Approach

The addressing scheme follows the device-type segmentation given in the project brief's example (Servers / User Devices / Printers / Wi-Fi), extended into full VLANs so each traffic type can be isolated and controlled by the firewall independently. Two additions beyond the brief's example are included as good practice for a real deployment:

- Guest Wi-Fi is split out from Staff Wi-Fi so visitors never touch the internal network.

- A management VLAN is added for the router, switch, and access points' own admin interfaces, so network hardware isn't reachable from ordinary user devices.

Each subnet is sized as a /24 (254 usable addresses) even though current device counts are far lower — this gives the company roughly 10x headroom to grow into each category without re-addressing later, at negligible cost.

# 2. Subnet Table

| VLAN | Subnet | Purpose | Gateway | Usable Range | Assignment Method |
| --- | --- | --- | --- | --- | --- |
| 10 | 192.168.10.0/24 | Servers & NAS | 192.168.10.1 | .10.2 – .10.254 | Static |
| 20 | 192.168.20.0/24 | Wired user devices (desktops & laptops) | 192.168.20.1 | .20.2 – .20.254 | DHCP (with reservations) |
| 30 | 192.168.30.0/24 | Network printers | 192.168.30.1 | .30.2 – .30.254 | Static / DHCP reservation |
| 40 | 192.168.40.0/24 | Staff Wi-Fi | 192.168.40.1 | .40.2 – .40.254 | DHCP |
| 50 | 192.168.50.0/24 | Guest Wi-Fi (isolated) | 192.168.50.1 | .50.2 – .50.254 | DHCP |
| 99 | 192.168.99.0/24 | Network management (router/switch/AP admin) | 192.168.99.1 | .99.2 – .99.254 | Static |

# 3. Specific Address Assignments

Based on the Hardware Inventory from Week 1:

| Device | VLAN | IP Address |
| --- | --- | --- |
| Router/firewall (gateway, all VLAN sub-interfaces) | — | 192.168.10.1 / .20.1 / .30.1 / .40.1 / .50.1 / .99.1 |
| Router/firewall management interface | 99 | 192.168.99.1 |
| Core switch management interface | 99 | 192.168.99.2 |
| NAS device | 10 | 192.168.10.10 |
| Wireless AP 1 | 99 | 192.168.99.10 |
| Wireless AP 2 | 99 | 192.168.99.11 |
| Wireless AP 3 | 99 | 192.168.99.12 |
| Network printer 1 | 30 | 192.168.30.10 |
| Network printer 2 | 30 | 192.168.30.11 |

Access points sit physically on VLAN 99 for their own management IP, while broadcasting the Staff SSID on VLAN 40 and the Guest SSID on VLAN 50 — standard practice for enterprise-grade Wi-Fi 6 APs.

# 4. DHCP Scope Plan

| VLAN | DHCP Pool | Reserved (static) Range |
| --- | --- | --- |
| 20 — User devices | .20.50 – .20.200 | .20.2 – .20.49 |
| 40 — Staff Wi-Fi | .40.50 – .40.250 | .40.2 – .40.49 |
| 50 — Guest Wi-Fi | .50.50 – .50.250 | n/a (no static devices) |

Servers, printers, and network management devices use static addressing or DHCP reservations (tied to MAC address) rather than the dynamic pool, so their IPs never change.

# 5. Inter-VLAN Access Rules (High-Level Firewall Policy)

| From \ To | VLAN 10 (Servers) | VLAN 20 (Users) | VLAN 30 (Printers) | VLAN 99 (Mgmt) | Internet |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 (Servers) | — | Limited (backup/services) | No | No | No (unless patching) |
| VLAN 20 (Users) | Yes (file shares) | — | Yes (printing) | No | Yes |
| VLAN 30 (Printers) | No | Yes (print jobs) | — | Yes (IT only) | No |
| VLAN 40 (Staff Wi-Fi) | Yes | Yes | Yes | No | Yes |
| VLAN 50 (Guest Wi-Fi) | No | No | No | No | Yes only |
| VLAN 99 (Mgmt) | Yes (admin) | No | Yes (admin) | — | Optional (firmware) |

This keeps guests fully isolated, protects network hardware from being reachable by ordinary staff devices, and restricts server access to only the traffic types that need it.

# 6. Growth Headroom

At 25 employees, current usage is roughly 25 user devices, 2 printers, 1 NAS, 3 APs, and 1 router/switch pair. Every subnet above is sized for up to 250+ devices, so Ubuntu Innovations can grow to 100+ staff without any re-addressing — only DHCP pool adjustments, if ever needed.